Statement of Work

Lucid Motors MPB EOL Production Test

# Revision History

| Date | Version | Author | Comment |
| --- | --- | --- | --- |
| 5/21/2020 | 1.0 | Victor Yang | First draft |
| 5/26/2020 | 1.1 | Mingkai Mu, Kayla Shi, Victor Yang | Add HVIL |
| 5/30/2020 | 1.2 | Shan Chai | Add EOL sequence. |
| 6/10/2020 | 1.3 | Kayla Shi, Victor Yang | Remove Hipot, and add equipment model |
| 7/02/2020 | 1.4 | Mingkai Mu, Victor Yang | Remove 18kW Load and add Compressor |
| 7/06/2020 | 1.5 | Ryan Lembitz | Change Chiller and Pressure |
| 09/17/2020 | 1.6 | Shan Chai/Shan Gao | Diagram, Communication |
| 09/22/2020 | 1.7 | Ryan Lembitz | Change DMM and Hipot brand |
| 09/23/2020 | 1.8 | Shan Gao/Victor Yang | Changed the SOW diagram |
| 09/28/2020 | 1.9 | Eric Kuntzelman | Change the NI relay module to a customized relay module with economizer |
| 09/29/2020 | 1.10 | Eric Kuntzelman/Shan Gao | Change the index number of contactor |

Purpose

Lucid Motors is developing and integration plan for MPB (Multifunction Power Box) production test stations.  MPB will provide on-board battery charger and DC/DC power function to all electrical vehicle required power following proposal. The Lucid team wants to use LabVIEW software to control all MPB function.

Test Station Summary

| HV component | Assembly level | Test station |
| --- | --- | --- |
| MPB HVIL | Assembly | HVIL Test |
| MPB Offset Tuning | Assembly | AC charging |
| MPB, AC power source, Electronic Load | Assembly | AC charging |
| MPB, 400V DC power source, Electronic load | Assembly | 400V DC charging |
| MPB, 800V DC power source, Electronic load | Assembly | 800V DC charging Power Line Conductivity |
| MPB, 1000V Power supply, Electronic load | Assembly | HAPS (Heater & AC Compressor) |
| MPB, 1000V Power supply, Electronic load | Assembly | DCDC |

GXV600M

https://www.gigavac.com/sites/default/files/catalog/spec_sheet/GXV600_DATASHEET_REV_3.pdf

ACS will need to control 4, 5, 6, 7, 8, 9, 10, 12, 14, 15.

| Equipment No. | Model | Link |
| --- | --- | --- |
| 4 | 2X E3633A, Agilent 12V Power supply, 4X contactor | http://literature.cdn.keysight.com/litweb/pdf/E3634-90001.pdf http://en.hf-relay.com/uploadfile/2019/0507/20190507032909220.pdf |
| 5 | 1000V/3kw DCDC Load Chroma 62150H-600 | http://www.chromaate.com/chroma/webdrive/catalogue/APP/TestMeasurement-PQG-E.pdf |
| 6 | E3633A, Agilent 12V Power supply | http://literature.cdn.keysight.com/litweb/pdf/E3634-90001.pdf |
| 7 | NI Ethernet Module IVN 8561 | Need to confirm if this is controlled via DSA. https://www.ni.com/en-us/support/model.ivn-8561.html |
| 8 | NI relay model NI-9485 | https://www.ni.com/pdf/manuals/374820a_02.pdf |
| 9 | Ni Voltage Module NI 9208 | https://www.ni.com/en-us/support/model.ni-9208.html |
| 10 | DC500V/50Kw Power Supply Chroma 62150H-600 | https://www.chromausa.com/pdf/62000H%20Datasheet.pdf |
| 11 | DMM, Chroma 12061 | https://www.chromausa.com/product/digital-multimeter-12061/?gclid=EAIaIQobChMIzKzbqai-6gIVUhh9Ch1nBwpJEAAYASAAEgI62_D_BwE |
| 12 | Regenerative Battery pack, Chroma 17040 | http://www.chromausa.com/pdf/17040-Regenerative%20Battery%20Pack-0519.pdf |
| 14 | AC compressor Electric Load Chroma 63205A-600-350 | www.chromaate.com/chroma/webdrive/catalogue/APP/TestMeasurement-PQG-E.pdf |
| 15 | Heater Electric Load Chroma 63208A-600-560 | www.chromaate.com/chroma/webdrive/catalogue/APP/TestMeasurement-PQG-E.pdf |
| 16 | Chroma 19053 Hipot Tester | https://www.tequipment.net/Chroma/19053/Hipot-Tester/?gclid=EAIaIQobChMIkInouMP96wIV6xatBh3OigaHEAAYASAAEgIHS_D_BwE |

In the EOL tests proposed below, it is required that LabVIEW will send a sequence number to MPB to setup the test mode for the corresponding test. Table below summaries communication that shall happen between LabVIEW and MPB.

Table 1: Ethernet Communication definition between MPB and LabVIEW

| Sequence Number | Used in Test |
| --- | --- |
| 1 | Close Relays of MPB in HiPot AC Test |
| 2 | Close Contactors of MPB in HiPot Batt-case Test |
| 3 | HIPOT Turn off All contactors |
| 4 | HVIL Test |
| 5 | Start- Offset Tuning |
| 6 | Read Offsets – Offset Tuning |
| 7 | AC Charge |
| 8 | DC 400V |
| 9 | DC 800V |
| 10 | HAPS-Heater output |
| 10 | HAPS-Compressor output |

Communication Protocol Automotive Ethernet:

MPB IP Address: 192.168.1.9

MPB VLAN: 1

MPB PORT: 11111

PC IP Address: 192.168.1.123

PC VLAN: 1

PC Port : 11112

Labview Sending Data Format :

0

SeqId

Labview Receving Data Format:  (Data size depends on Payload size)

| 0 | 1 | 2 |  |
| --- | --- | --- | --- |
| SeqId | Payload Size | Data |  |

Integration Proposal

Application Flashing

- LabVIEW shall turn on the 12V power supply.

- LabVIEW shall send signal to DSA to achieve software flashing via UDS.

Hipot Test

HIPOT AC TEST

- Connect Chroma 19053 Hipot tester output to AC input

- Connect Hipot tester input to Pack connector

- LABVIEW shall sent out sequence number 1 to MPB to close the AC relays in MPB.

- Send out signal to turn on Hipot tester.

  - Test setting (3000V, 10s Ramping, 30s holding, current<2mA)

  - IR resistor should be above 10M

- Record Pass/Fail

HIPOT Battery-Case Test

- Connect Chroma 19053 Hipot tester output to MPB case

- LabVIEW shall send out signal to MPB sequence number 2 to close the internal contactors and relays.

- Send out signal to turn on Hipot tester

  - Test setting (3000V, 10s Ramping, 30s holding, current<2mA)

  - IR resistor should be above 10M

- Record Pass/Fail

  - current<2mA)

  - IR resistor should be above 10M

- Record Pass/Fail

- Turn off power

- LAVIEW shall send out signal to MPB sequence number 3 to disconnect the internal contactors and relays.

MPB Automotive Ethernet Interface

All MPB communication shall be integrated to LabVIEW test program

Test Software

- LabVIEW

Test Data Requirements

| Test input |  |
| --- | --- |

HVIL Test

Connection and Test sequence

- Insert connectors for MPB Inverter output, AC input, DC input, DCDC, and Pack connectors

- Insert connectors for LV connector, Heater, and Electronic AC compressor connectors

- Record continuity between pin 5 and pin 6 (PT HVIL) of the LV connector, and use LabVIEW monitor resistance.

- The LabVIEW shall send the sequence number 4 to start HVIL Tester. MPB will conduct self-test and send the result back to LabVIEW.

LabVIEW Receiving Data Format:

| 0 [SeqId] | 1[Payload Size] | 2[Data] |
| --- | --- | --- |
| 4 | 1 | 1/0 |

- The LabVIEW shall display the result.

- Check Pass/Fail and turn off power

| Test output | Pass/Fail |
| --- | --- |
| Test log | Yes |

MPB Automotive Ethernet Interface

All MPB communication shall be integrated to LabVIEW test program

Test Software

- LabVIEW

Test Data Requirements

| Test input |  |
| --- | --- |
| Test output | Pass/Fail |
| Test log | Yes |

Sensor Offset Tuning and Calibration

This procedure has to be conducted before any of the following high power test.

Connection and Test sequence

- Use LabVIEW to send signal to turn on LV Harness with 12V power supply.

- Use LabVIEW to send the EOL sequence number 5 to MPB to start the sequence.

- Use LabVIEW to send the EOL sequence number 6 to MPB to get the offset values.

- Connect the DC supply to the battery port, through contactors.

- Use LabVIEW to send the EOL sequence number ??? to get the data

- Control DC supply to provide 800V to the MPB battery port

- Use LabVIEW to read the data in MPB, and compare with the reference for pass/fail.

- Turn off the power supply, wait until the internal voltage go low.

- Use LabVIEW to send command to disconnect contactors in the MPB.

- Labview Receiving Data Format :

| 0 [SeqId] | 1[Payload Size] | 2[Data] |
| --- | --- | --- |
| 4 | 1 | 1/0 |

- LabVIEW shall display and compare it with pre-defined threshold to indicate it pass or fail.

MPB Automotive Ethernet Interface

All MPB communication shall be integrated to LabVIEW test program

Test Software

- LabVIEW

Test Data Requirements

| Test input |  |
| --- | --- |
| Test output | Pass/Fail |
| Test log | Yes |

AC charging

Connection and Test sequence

- Turn on AIR1000 chiller with 20psi and 20C test condition and connect coolant with MPB through water pipe during the test.

- Set Regenerative Battery Pack (model 17040) to 800V/18kW as an Electronic Load (50Ohm) and connect to MPB Pack connector by switching on the corresponding contactors from LabVIEW.

- TURN on 925V power supply.

- Connect AC source to EVSE, and insert charging gun to AC connector

- Use LabVIEW to send signal to turn on LV Harness with 12V power supply and all NI sensor detectors.

- Use LabVIEW send Ethernet signal to start the EOL sequence 7 and also send/receiving the following Ethernet Signal

| AC Charge Test |  |  |
| --- | --- | --- |
| Sending Signals | Value | Unit |
| IPT_State | 2 | Enum |
| IBMU_ChrgCurrTar | 19 | A |
| IBMU_ChrgVoltTarg | 924 | V |
| IMPB_FltClear (TBD) | 0 | Bool |
| Reading Signals | Value | Unit |
| IMPB_BattCurrDAB1 |  | A |
| IMPB_BattCurrDAB2 |  | A |
| IMPB_BattVoltgHiRes |  | V |
| IMPB_DCbusVoltg |  | V |
| IMPB_BattPortPower |  | was |
| IMPB_HwFltCode |  | uint32 |
| IMPB_SwFltCode (TBD) |  | uint32 |
| IMPB_HwFltFlag |  | Bool |

- Use LabVIEW to send signal to turn on AC power to start to charge.

- MPB will send output DC voltage, DC current and fault code to LabVIEW via Ethernet.

- LabVIEW shall display and compare it with pre-defined threshold to indicate it pass or fail.

- Check Pass/Fail and turn off power

MPB Automotive Ethernet Interface

All MPB communication shall be integrated to LabVIEW test program

Test Software

- LabVIEW

Test Data Requirements

| Test input |  |
| --- | --- |
| Test output | Pass/Fail |
| Test log | Yes |

DC 400V charging

Connection and Test sequence

- Set Regenerative Battery Pack (model 17040) to 1000V/50kW as an Electronic Load and connect to MPB Pack connector by switching on the corresponding contactors from LabVIEW.

- Use LabVIEW to send signal to turn on LV Harness with 12V power supply and all NI sensor detectors.

- Start PLC (Power Line Communication) emulator software on PC.

- Use LabVIEW to send the EOL sequence number 8 and send/receive the following signal.

| Sending Signals | Value | Unit |
| --- | --- | --- |
| IPT_State | 2 | Enum |
| IBMU_ChrgCurrTar | 50 | Bool |
| IBMU_ChrgVoltTarg | 924 | V |
| IMPB_FltClear (TBD) | 0 | Bool |
| Reading Signals | Value | Unit |
| IMPB_BattCurrDAB1 |  | A |
| IMPB_BattCurrDAB2 |  | A |
| IMPB_BattVoltgHiRes |  | V |
| IMPB_DCbusVoltg |  | V |
| IMPB_BattPortPower |  | was |
| IMPB_HwFltCode |  | uint32 |
| IMPB_SwFltCode (TBD) |  | uint32 |
| IMPB_HwFltFlag |  | Bool |

- PLC emulator on PC will turn on 500V/50kW DC power to start to charge.

- MPB will send output DC voltage, DC current and fault code to LabVIEW via Ethernet.

- LabVIEW shall display and compare it with pre-defined threshold to indicate it pass or fail.

- Check Pass/Fail and turn off power

MPB Automotive Ethernet Interface

All MPB communication shall be integrated to LabVIEW test program.

Test Software

- LabVIEW

Test Data Requirements

| Test input |  |
| --- | --- |
| Test output | Pass/Fail |
| Test log | Yes |

DC 800V Charging Power Line Conductivity

The main purpose of this test is to test the conductivity of 800V power line on MPB instead of 800V charging.

Connection and Test sequence

- Switch to connect MPB Pack connector to short connection by switching on the corresponding contactor from LabVIEW.

- Use LabVIEW to set 50kW DC power to 2V/100A limit

- Use LabVIEW to send signal to turn on LV Harness with 12V power supply and NI sensor detectors.

- Use LabVIEW to send the EOL sequence number 9 to MPB to start the sequence.

- Use LabVIEW to send signal to turn on 50kW power to start outputting voltage.

- LabVIEW shall measure DC input voltage and the conduction resistance and compare the conduction resistance with pre-defined threshold to indicate it pass or fail.

- Check Pass/Fail and turn off power

MPB Automotive Ethernet Interface

All MPB communication shall be integrated to LabVIEW test program.

Test Software

- LabVIEW

Test Data Requirements

| Test input |  |
| --- | --- |
| Test output | Pass/Fail |
| Test log | Yes |

HAPS-Heater output

Connection and Test sequence

- Set Regenerative Battery Pack (model 17040) to 925V/10kW as a Power Source and connect to MPB Pack connector by switching on the corresponding contactor from the LabVIEW.

- Connect MPB Heater output to 400V/7kW with current limit 20A Electronic Load.

- Use LabVIEW to send signal to turn on LV Harness with 12V power supply, all NI sensor detectors.

- Use the LabVIEW to start 50kW regen/load to output 925V.

- Use LabVIEW to send the EOL sequence number 10 and also send/receive the following signal

| Haps Test |  |  |
| --- | --- | --- |
| Sending Signals | Value | Unit |
| IMPB_HAPSOnOff | 1 | Enum |
| IMPB_HAPSFaultClear | 0 | Bool |
| Reading Signals | Value | Unit |
| IMPB_HAPSCurr |  | A |
| IMPB_HAPSPwr |  | was |
| IMPB_HAPSVoltgOut |  | V |
| IMPB_HAPSFltFlag |  | Bool |
| IMPB_HAPSFltCode |  | uint8 |

- MPB will send output DC voltage, DC current and fault code to LabVIEW via Ethernet.

- LabVIEW shall display and compare it with pre-defined threshold to indicate it pass or fail.

- Check Pass/Fail and turn off power

MPB Automotive Ethernet Interface

All MPB communication shall be integrated to LabVIEW test program

Test Software

- LabVIEW

Test Data Requirements

| Test input |  |
| --- | --- |
| Test output | Pass/Fail |
| Test log | Yes |

HAPS-Compressor output

Connection and Test sequence

- Set Regenerative Battery Pack (model 17040) to 925V/10kW as a Power Source and connect to MPB Pack connector by switching on the corresponding contactor from the LabVIEW.

- Connect MPB Compressor output to 400V/4kW with current limit 20A Electronic Load.

- Use LabVIEW to send signal to turn on LV Harness with 12V power supply, all NI sensor detectors.

- Use the LabVIEW to start 550V/10kW power supply to output 550V.

- Use LabVIEW to send the EOL sequence number 10 and also send/receive the following signal

| Haps Test |  |  |
| --- | --- | --- |
| Sending Signals | Value | Unit |
| IMPB_HAPSOnOff | 1 | Enum |
| IMPB_HAPSFaultClear | 0 | Bool |
| Reading Signals | Value | Unit |
| IMPB_HAPSCurr |  | A |
| IMPB_HAPSPwr |  | was |
| IMPB_HAPSVoltgOut |  | V |
| IMPB_HAPSFltFlag |  | Bool |
| IMPB_HAPSFltCode |  | uint8 |

- MPB will send output DC voltage, DC current and fault code to LabVIEW via Ethernet.

- LabVIEW shall display and compare it with pre-defined threshold to indicate it pass or fail.

- Check Pass/Fail and turn off power

MPB Automotive Ethernet Interface

All MPB communication shall be integrated to LabVIEW test program

Test Software

- LabVIEW

Test Data Requirements

| Test input |  |
| --- | --- |
| Test output | Pass/Fail |
| Test log | Yes |

DCDC output

Connection and Test sequence

- Set Regenerative Battery Pack (model 17040) to 925V/10kW as a Power Source and connect to MPB Pack connector by switching on the corresponding contactor from the LabVIEW.

- Set Electronic Load to 925V/3kW, and Connect MPB DCDC output.

- Use LabVIEW to send signal to turn on LV Harness with 12V power supply, all NI sensor detectors.

- Use the LabVIEW to start 925V/10kW power supply to output 925V.

- LabVIEW shall read the voltage and current from E-Load.

- LabVIEW shall display and compare it with pre-defined threshold to indicate it pass or fail.

- Check Pass/Fail and turn off power

- Turn off the chiller to complete EOL test

MPB Automotive Ethernet Interface

All MPB communication shall be integrated to LabVIEW test program

Test Software

- LabVIEW

Test Data Requirements

| Test input |  |
| --- | --- |
| Test output | Pass/Fail |
| Test log | Yes |